VDA 5050 v3
ROX-Diff–Crane
Commissioning Runbook

Neobotix ROX-Diff first test, Raspberry Pi master, crane adapter and coordinated no-load handover

Prepared for the VDA5050 Paper project
21 July 2026
DTLabOpen, ROX boot-autostart, ROS-interface and df_map revision

Commissioning document — not a safety certification. Physical safety systems and approved local procedures remain authoritative.

# Run sequence

Execute the parts in order. The first combined crane–AMR movement occurs only in Part H, after independent tests pass.

Part A: Prepare the Raspberry Pi master

Part B: Prepare the Neobotix ROX-Diff

Part C: Verify the existing ROX map and Nav2; remap only if required

Part D: Capture waypoints and generate the VDA order

Part E: Test the ROX adapter without robot movement

Part F: First real ROX motion

Part G: Configure and test the crane adapter

Part H: Coordinated ROX–crane no-load test

Part I: Optional systemd services

Part J: Fast troubleshooting

Manual fields: Complete Appendix A on the real equipment. Network addresses, ROS Jazzy/Cyclone DDS settings, boot-time ROX bringup arguments, currently visible hardware topics and the existing df_map file paths are recorded below. Prefix lengths, Nav2 action/TF after navigation startup, map backup, physical dimensions and PLC credentials must still be verified on site.

Project roots used in this guide: Raspberry Pi ~/VDA5050-Paper-Dev; ROX-Diff ~/Projects/VDA5050-Paper-Dev

Raspberry Pi MQTT address on DTLabOpen: 192.168.50.115

Protocol identity: VDA 5050 3.0.0

This runbook starts with the Neobotix ROX-Diff alone, then the Raspberry Pi master, then the crane, and only then the coordinated no-load handover. Do not skip directly to combined movement.

## Safety boundary

The MQTT/Flask/VDA software is not safety-rated. The ROX safety controller, scanners, emergency stops, motor controller, crane PLC, crane emergency stop and local operating procedures remain authoritative. Keep the work area clear, use reduced commissioning speeds, keep emergency stops reachable, and test without a payload before any loaded test.

## Current network architecture

The Raspberry Pi is dual-homed. Its DTLabOpen Ethernet interface is the direct MQTT/Flask path to the ROX-Diff, while its Ilmatar Wi-Fi interface remains a separate connection for the private Ilmatar network. The ROX-Diff and Pi are direct peers on DTLabOpen; no intermediate training network or NAT path is used.

Known and verified values:

Pi Ethernet on DTLabOpen: 192.168.50.115 (verify the actual prefix length with ip -br address).

Pi Wi-Fi on Ilmatar: 192.168.0.116 (verify the actual prefix length and gateway).

DTLabOpen gateway: 192.168.1.1 (unchanged). Keep the Pi default-route behavior exactly as verified on the working system; do not change it merely for this project.

ROX-Diff DTLabOpen address: 192.168.50.50 (static; verify the actual prefix length and interface name).

ROX-Diff network connection: DTLabOpen with static IPv4 address 192.168.50.50.

ROX-Diff internal control/safety network stays separate. Do not renumber scanner, FlexiSoft, relay-board or robot-internal devices merely to match the training network.

The normal communication path is direct over DTLabOpen: ROX-Diff 192.168.50.50 <-> Raspberry Pi 192.168.50.115. MQTT uses 192.168.50.115:1883 and the Flask interface uses 192.168.50.115:5000. Because both devices are on the same DTLabOpen network, no NAT, port-forward or additional route is required for normal bidirectional communication.

Verify the Pi interfaces and routes before starting Mosquitto:

ip -br address
ip route
nmcli -f NAME,DEVICE,TYPE,IP4.ADDRESS,IP4.GATEWAY connection show --active
ip route get 192.168.1.1

The route table must contain only one preferred default route, through Ilmatar. If the DTLabOpen NetworkManager profile also installs a default route, disable it without changing the static address:

sudo nmcli connection modify "<DTLabOpen profile>" \
  ipv4.never-default yes ipv6.never-default yes
sudo nmcli connection up "<DTLabOpen profile>"

Do not move the Pi Ethernet interface to 192.168.0.x while its Ilmatar Wi-Fi is also 192.168.0.x. Two active interfaces in the same subnet create ambiguous routing and ARP behavior.

# Part A — Prepare the Raspberry Pi master

## A1. Put the project on the Pi

Using Git:

cd ~ git clone https://github.com/Huxyshuu/VDA5050-Paper-Dev.git cd ~/VDA5050-Paper-Dev git pull --ff-only git rev-parse --short HEAD

Or copy the supplied updated ZIP to the Pi and extract it:

cd ~ unzip VDA5050-Paper-Dev-updated.zip mv VDA5050-Paper-Dev-updated VDA5050-Paper-Dev cd ~/VDA5050-Paper-Dev

Confirm the expected files:

pwd ls README.md fleet_control/master_control.py scripts/run_master_control.sh

Repository preflight: the public main branch inspected on 21 July 2026 still contained a tracked crane credential file and commented-out credential ignore rules. Before crane work, follow the credential-remediation steps in Part G2. Do not copy a credential-bearing checkout to additional devices.

Record the exact revision used for every test:

git status --short
git log -1 --oneline

## A2. Install the broker and basic tools

sudo apt update sudo apt install -y \ mosquitto mosquitto-clients \ python3 python3-venv python3-pip \ netcat-openbsd curl unzip git sudo systemctl enable --now mosquitto

## A3. Create the Python environment

cd ~/VDA5050-Paper-Dev python3 -m venv .venv source .venv/bin/activate python -m pip install --upgrade pip python -m pip install -r fleet_control/requirements.txt

## A4. Configure the Pi master

cd ~/VDA5050-Paper-Dev cp -n configs/fleet_control.env.example configs/fleet_control.env nano configs/fleet_control.env

Fill or verify at least:

VDA_MQTT_HOST=192.168.50.115 VDA_MQTT_PORT=1883 VDA_DEFAULT_MAP_ID=df_map CRANE_TOPIC_ROOT=vda5050/v3/konecranes/ilmatar_1 ROX_TOPIC_ROOT=vda5050/v3/neobotix/rox_diff_1 CRANE_AUTO_RELEASE_ACTION_ID=action4 CRANE_MANUAL_RELEASE_ACTION_ID=action6 CRANE_SAFE_LIFT_ACTION_ID=action7 ROX_HOLD_ACTION_ID=rox_hold_at_crane

Do not fill ROX_INIT_X, ROX_INIT_Y or ROX_INIT_THETA with DBot values. The order generator fills them after real ROX waypoints have been captured.

Use 192.168.50.115 because it is the Pi address directly reachable by the ROX-Diff on DTLabOpen. Do not substitute the Ilmatar Wi-Fi address 192.168.0.116 in the ROX adapter configuration.

## A5. Permit MQTT access on the isolated lab network

Inspect the supplied broker example:

cat deploy/mosquitto/vda5050-lab.conf.example

Install it only on a trusted, isolated commissioning network:

sudo cp deploy/mosquitto/vda5050-lab.conf.example \ /etc/mosquitto/conf.d/vda5050-lab.conf sudo systemctl restart mosquitto sudo systemctl status mosquitto --no-pager ss -ltnp ¦ grep 1883

Anonymous unencrypted MQTT must not be exposed to a general company, campus or Internet-facing network. Add authentication and TLS before production deployment.

Because the Pi also connects to Ilmatar, confirm that anonymous MQTT is not unintentionally reachable from the Ilmatar interface. Bind or firewall the lab listener to loopback and the DTLabOpen-facing address 192.168.50.115, or enable authentication/TLS. Test both intended and unintended interfaces before motion commissioning.

ss -ltnp | grep 1883
nc -vz 192.168.50.115 1883
# From an Ilmatar-side host, TCP 1883 should be blocked unless explicitly approved.

## A6. Run repository checks

cd ~/VDA5050-Paper-Dev ./scripts/run_static_checks.sh

All checks should end in PASS. These checks validate source syntax, schemas and generated test messages; they do not test ROS, Nav2, the PLC or physical hardware.

Also verify that no credential file is tracked and no local secret appears in the working-tree diff:

git ls-files crane_edge/access.txt legacy/RaspberryPI/accesscode_url.txt
git status --short
git diff --check

## A7. Start the master manually

Terminal Pi-1:

cd ~/VDA5050-Paper-Dev source .venv/bin/activate ./scripts/run_master_control.sh

Terminal Pi-2, inspect the runtime:

curl http://127.0.0.1:5000/runtime ¦ python3 -m json.tool

Terminal Pi-3, monitor all VDA traffic:

mosquitto_sub -h 127.0.0.1 -t 'vda5050/v3/#' -v

From another computer, open:

http://192.168.50.115:5000

At this stage the master may report that the generated ROX order is missing. That is expected until Part D.

# Part B — Prepare the Neobotix ROX-Diff

## B1. Copy or clone the project on the robot computer

cd ~ git clone https://github.com/Huxyshuu/VDA5050-Paper-Dev.git cd ~/VDA5050-Paper-Dev git pull --ff-only git rev-parse --short HEAD

For the supplied ZIP, copy and extract it instead, then use the ROX project path ~/Projects/VDA5050-Paper-Dev.

## B2. Identify the installed ROS and Neobotix workspace

printenv ROS_DISTRO ls /opt/ros find ~ -maxdepth 3 -path '*/install/setup.bash' -print

The current site record and helper-script default use:

ROS_DISTRO=jazzy NEOBOTIX_WS=$HOME/ros2_workspace

Verify these values on the delivered robot and set different values if required:

export ROS_DISTRO=<installed_ros_distribution> export NEOBOTIX_WS=<path_to_neobotix_workspace>

Example:

export ROS_DISTRO=jazzy export NEOBOTIX_WS="$HOME/ros2_workspace" export RMW_IMPLEMENTATION=rmw_cyclonedds_cpp export ROS_DOMAIN_ID=0

## B3. Verify the native Neobotix packages before building this project

source /opt/ros/$ROS_DISTRO/setup.bash source "$NEOBOTIX_WS/install/setup.bash" ros2 pkg prefix rox_bringup ros2 pkg prefix rox_navigation

List the exact Python launch files installed on this robot. The delivered bringup files use names such as bringup_launch.py, so search for all *.py files rather than only *.launch.py:

find "$(ros2 pkg prefix rox_bringup)/share/rox_bringup/launch" -maxdepth 1 -type f -name '*.py' -printf '%f\n' ¦ sort find "$(ros2 pkg prefix rox_navigation)/share/rox_navigation/launch" -maxdepth 1 -type f -name '*.py' -printf '%f\n' ¦ sort

The installed files have now been verified as bringup_launch.py, mapping.launch.py, navigation.launch.py and localization_neo.launch.py. Confirm their accepted arguments with --show-arguments:

ros2 launch rox_navigation mapping.launch.py --show-arguments ros2 launch rox_navigation navigation.launch.py --show-arguments ros2 launch rox_navigation localization_neo.launch.py --show-arguments

The physical bringup command is already embedded in the robot boot script. Use this command only to inspect its supported arguments; do not start a second bringup instance:

ros2 launch rox_bringup bringup_launch.py --show-arguments

Record the verified launch arguments, boot-autostart mechanism and observed ROS interfaces in docs/SITE_CONFIGURATION_CHECKLIST.md.

## B4. Build this repository as a separate overlay

Do not copy the adapter into or overwrite Neobotix packages. The project remains a separate overlay under ~/Projects/VDA5050-Paper-Dev/ros2_ws.

cd ~/Projects/VDA5050-Paper-Dev export ROS_DISTRO=${ROS_DISTRO:-jazzy} export NEOBOTIX_WS=${NEOBOTIX_WS:-$HOME/ros2_workspace} export AMENT_TRACE_SETUP_FILES="" ./scripts/build_rox_overlay.sh

In every new ROX terminal, source the native Neobotix workspace and then the project overlay. The same sequence may be placed in ~/.bashrc after the overlay has been built:

export ROS_DISTRO=${ROS_DISTRO:-jazzy} export NEOBOTIX_WS=${NEOBOTIX_WS:-$HOME/ros2_workspace} source /opt/ros/$ROS_DISTRO/setup.bash source "$NEOBOTIX_WS/install/setup.bash" source ~/Projects/VDA5050-Paper-Dev/ros2_ws/install/setup.bash

Confirm the adapter is visible:

ros2 pkg prefix rox_vda5050_adapter ros2 pkg executables rox_vda5050_adapter

## B5. Verify Pi network and MQTT from ROX

ping -c 3 192.168.50.115 nc -vz 192.168.50.115 1883

Install MQTT client tools if needed:

sudo apt update sudo apt install -y mosquitto-clients netcat-openbsd

Run the supplied check:

cd ~/Projects/VDA5050-Paper-Dev ./scripts/check_pi_mqtt_from_rox.sh 192.168.50.115 1883

On the Pi, confirm that the message arrives:

mosquitto_sub -h 127.0.0.1 \ -t 'vda5050/v3/commissioning/ping' -C 1 -v

The ROX-Diff should be connected directly to DTLabOpen with static address 192.168.50.50. Confirm that traffic to the Pi address 192.168.50.115 uses the DTLabOpen-facing interface:

ip -br address
ip route
ip route get 192.168.50.115
ping -c 3 192.168.50.115
nc -vz 192.168.50.115 1883

Acceptance requires bidirectional IP connectivity between 192.168.50.50 and 192.168.50.115, plus a successful ROX-to-Pi MQTT test on TCP port 1883.

## B6. Verify the boot-time native ROX bringup

The native ROX hardware stack is already launched automatically whenever the robot boots. The boot hook runs ROS_AUTOSTART.sh with the following content:

source ~/ros2_workspace/install/setup.bash sleep 2 ros2 launch rox_bringup bringup_launch.py rox_type:=diff imu_enable:=True use_d435:=True enable_io_board:=True

This starts the ROX-Diff hardware drivers with IMU, RealSense D435 and I/O board support enabled. The scanner type remains the bringup default nanoscan and the robot namespace remains empty because neither is explicitly passed.

Do not run bringup_launch.py again in another terminal during normal commissioning. A second instance can create duplicate nodes and TF publishers or compete for cameras, scanners, relay-board interfaces and other hardware resources. Instead, open a new terminal and verify the already-running boot process:

source /opt/ros/$ROS_DISTRO/setup.bash source "$NEOBOTIX_WS/install/setup.bash" cd ~/Projects/VDA5050-Paper-Dev ./scripts/check_rox_ros_interfaces.sh

The current interface audit confirms the native hardware topics below. The Nav2 action and map-to-base transform are intentionally checked again only after mapping/localization/navigation is launched in Part C:

ros2 node list ¦ sort ros2 topic list -t ¦ sort ros2 topic echo /odom --once ros2 topic echo /scan --once ros2 topic echo /emergency_stop_state --once ros2 topic echo /safety_state --once ros2 interface show neo_msgs2/msg/EmergencyStopState ros2 interface show neo_msgs2/msg/SafetyState ros2 run tf2_ros tf2_echo odom base_link

Current result interpretation: /tf, /tf_static, /odom, /battery_state, /emergency_stop_state, /safety_state and /scan are present with the expected message types. The blank "Required Nav2 action" and blank "map->base_link transform sample" are not bringup failures; Nav2 and map localization were not running yet. Do not launch the VDA adapter until these two entries appear after Part C and the physical emergency-stop/scanner behavior has been tested.

# Part C — Verify the existing ROX map and Nav2; remap only if required

## C1. Verify the existing df_map files

MAP_DIR="/home/neobotix/ros2_workspace/install/rox_navigation/share/rox_navigation/maps" ls -l "$MAP_DIR/df_map.yaml" "$MAP_DIR/df_map.pgm" grep -E '^(image¦resolution¦origin):' "$MAP_DIR/df_map.yaml"

The currently selected site map is df_map. Both files are installed under /home/neobotix/ros2_workspace/install/rox_navigation/share/rox_navigation/maps. Verify that the YAML and PGM exist together and that the YAML image entry resolves to df_map.pgm:

ros2 launch rox_navigation navigation.launch.py --show-arguments ros2 launch rox_navigation localization_neo.launch.py --show-arguments

The navigation launch must receive the absolute df_map.yaml path. Do not use the package default neo_workshop.yaml for this commissioning site.

## C2. Back up the existing map before testing

The map currently lives inside the workspace install tree. Treat that location as installed/generated content: a future workspace rebuild or package reinstall may replace it. Copy both files together to a separate backup location before commissioning.

Terminal ROX-2:

MAP_DIR="/home/neobotix/ros2_workspace/install/rox_navigation/share/rox_navigation/maps" BACKUP_DIR="$HOME/maps/df_map_backup" mkdir -p "$BACKUP_DIR" cp -a "$MAP_DIR/df_map.yaml" "$MAP_DIR/df_map.pgm" "$BACKUP_DIR/" ls -l "$BACKUP_DIR/df_map.yaml" "$BACKUP_DIR/df_map.pgm"

Keep the YAML and PGM as a matched pair. If the YAML contains a relative image value such as image: df_map.pgm, preserve the filenames and directory relationship in the backup.

Do not remap merely because Nav2 is not yet running. First test localization against df_map and check the TF tree, lidar alignment and costmaps.

## C3. Start localization and navigation with df_map

source /opt/ros/$ROS_DISTRO/setup.bash source "$NEOBOTIX_WS/install/setup.bash" ros2 launch rox_navigation navigation.launch.py \ rox_type:=diff \ use_amcl:=True \ map:=/home/neobotix/ros2_workspace/install/rox_navigation/share/rox_navigation/maps/df_map.yaml

The native hardware bringup from ROS_AUTOSTART.sh must remain running. Start only the navigation/localization stack in this terminal; do not launch bringup_launch.py again.

## C4. Optional: recreate the map only if df_map is unsuitable

Use mapping.launch.py only if the environment has materially changed or df_map cannot localize reliably after its path and YAML image reference have been verified. The mapping launch accepts autostart, use_lifecycle_manager, use_sim_time and slam_params_file; it does not accept rox_type or frame_type.

Terminal ROX-2:

source /opt/ros/$ROS_DISTRO/setup.bash source "$NEOBOTIX_WS/install/setup.bash" ros2 launch rox_navigation mapping.launch.py

When remapping, keep the boot-autostarted native hardware bringup running and launch only mapping.launch.py. Save the replacement YAML and PGM together, back them up, and then pass the new absolute YAML path to navigation.launch.py. Do not pass rox_type or frame_type to mapping.launch.py.

## C5. Verify Nav2 and the previously missing interfaces before VDA

ros2 action list -t ¦ grep -E 'navigate_to_pose¦NavigateToPose' ros2 run tf2_ros tf2_echo map base_link cd ~/Projects/VDA5050-Paper-Dev ./scripts/check_rox_ros_interfaces.sh

In RViz:

1.  Set the initial pose.

2.  Confirm lidar overlays the map.

3.  Confirm global and local costmaps.

4.  Send at least three normal Nav2 goals.

5.  Repeat the crane approach from different directions.

6.  Confirm position and orientation repeatability.

7.  Confirm the robot can stop and depart without violating scanner fields.

The VDA adapter must not be used to compensate for a poor map, incorrect TF, wrong footprint, unstable localization or untuned Nav2 controller.

# Part D — Capture waypoints and generate the VDA order

## D1. Create the site waypoint file on ROX

cd ~/VDA5050-Paper-Dev cp -n configs/rox_waypoints.yaml.example configs/rox_waypoints.yaml

Keep configured: false until all poses have been captured and individually tested.

## D2. Capture each pose

Source the project overlay in the terminal used for capture:

source /opt/ros/$ROS_DISTRO/setup.bash source "$NEOBOTIX_WS/install/setup.bash" source ~/Projects/VDA5050-Paper-Dev/ros2_ws/install/setup.bash cd ~/Projects/VDA5050-Paper-Dev

At each physical pose, wait for localization to settle and run the corresponding command:

ros2 run rox_vda5050_adapter capture_waypoint \ --name home \ --output "$PWD/configs/rox_waypoints.yaml" \ --map-id df_map

ros2 run rox_vda5050_adapter capture_waypoint \ --name short_test \ --output "$PWD/configs/rox_waypoints.yaml" \ --map-id df_map

ros2 run rox_vda5050_adapter capture_waypoint \ --name crane_handover \ --output "$PWD/configs/rox_waypoints.yaml" \ --map-id df_map

ros2 run rox_vda5050_adapter capture_waypoint \ --name warehouse_dropoff \ --output "$PWD/configs/rox_waypoints.yaml" \ --map-id df_map

Review the file:

cat configs/rox_waypoints.yaml

## D3. Verify every captured pose with normal Nav2

For each waypoint, move away and return using RViz/Nav2 several times. Verify:

the robot center and full footprint;

payload clearance;

final yaw;

scanner fields;

crane reach and hook/load alignment;

departure path;

practical position/orientation tolerances.

Then edit:

nano configs/rox_waypoints.yaml

Set:

configured: true

Do not set this flag merely to bypass the generator check.

## D4. Copy the waypoint file to the Pi

From ROX:

scp ~/Projects/VDA5050-Paper-Dev/configs/rox_waypoints.yaml \ <PI_USER>@192.168.50.115:/home/<PI_USER>/VDA5050-Paper-Dev/configs/

Replace <PI_USER> with the real Pi username.

This SCP command runs directly from the ROX-Diff on DTLabOpen to the Pi at 192.168.50.115. If it fails, verify SSH is enabled on the Pi, test TCP port 22 and confirm the Pi username and destination path.

## D5. Generate the short test order on the Pi

cd ~/VDA5050-Paper-Dev source .venv/bin/activate python3 scripts/generate_rox_order.py \ --waypoints configs/rox_waypoints.yaml \ --route examples/routes/rox_short_motion_test.yaml \ --output examples/orders/order_rox_diff_v3.json \ --update-fleet-env configs/fleet_control.env ./scripts/run_static_checks.sh

Inspect the generated order and initial pose:

python3 -m json.tool examples/orders/order_rox_diff_v3.json ¦ less grep '^ROX_INIT_' configs/fleet_control.env

Restart the Pi master so it reloads the order and environment:

# In the master terminal: Ctrl+C cd ~/VDA5050-Paper-Dev source .venv/bin/activate ./scripts/run_master_control.sh

# Part E — Test the ROX adapter without robot movement

## E1. Keep native bringup and Nav2 running

Use the native commands verified in Parts B and C.

## E2. Start the VDA adapter in dry-run mode

Terminal ROX-3:

cd ~/Projects/VDA5050-Paper-Dev export ROS_DISTRO=${ROS_DISTRO:-jazzy} export NEOBOTIX_WS=${NEOBOTIX_WS:-$HOME/ros2_workspace} export VDA_MQTT_HOST=192.168.50.115 export VDA_MAP_ID=df_map ./scripts/run_rox_adapter_dry.sh

Expected first observations on the Pi MQTT monitor:

.../connection ONLINE .../factsheet retained factsheet template .../state periodic v3 state

Keep VDA_MQTT_HOST=192.168.50.115. The ROX-Diff and Pi communicate directly on DTLabOpen, so no port-forwarding or additional routing is required for VDA messages.

## E3. Request the factsheet and send the ROX order

On the Pi:

curl -X POST http://127.0.0.1:5000/factsheet/rox curl -X POST http://127.0.0.1:5000/order/rox curl http://127.0.0.1:5000/runtime ¦ python3 -m json.tool

The dry run must progress through simulated nodes and stop with the configured holdPose action running. Release it:

curl -X POST http://127.0.0.1:5000/release_hold

Test instant actions individually:

curl -X POST http://127.0.0.1:5000/pause/rox curl -X POST http://127.0.0.1:5000/resume/rox curl -X POST http://127.0.0.1:5000/cancel/rox curl -X POST http://127.0.0.1:5000/automatic

Acceptance criteria:

no schema errors;

correct manufacturer, serial number and version;

order accepted only when identity, map and sequence are valid;

holdPose remains active until released;

pause/resume/cancel action states reach a terminal state;

initializePosition uses ROS frame map and the configured logical mapId;

no physical motion occurs.

# Part F — First real ROX motion

## F1. Stop the dry-run adapter

Press Ctrl+C in ROX-3.

## F2. Confirm the short order is active

On the Pi:

grep '^ROX_ORDER_JSON_PATH=' configs/fleet_control.env python3 -m json.tool examples/orders/order_rox_diff_v3.json ¦ head -80

The route should be the two-node commissioning route using home and short_test.

## F3. Position and localize ROX at home

Use RViz and the native robot tools. Confirm that map -> base_link is within the waypoint tolerance. The adapter intentionally rejects a real order if the robot is not near the first node.

## F4. Start real navigation mode

Terminal ROX-3:

cd ~/Projects/VDA5050-Paper-Dev export ROS_DISTRO=${ROS_DISTRO:-jazzy} export NEOBOTIX_WS=${NEOBOTIX_WS:-$HOME/ros2_workspace} export VDA_MQTT_HOST=192.168.50.115 export VDA_MAP_ID=df_map ./scripts/run_rox_adapter_real.sh

## F5. Send only the ROX order

On the Pi:

curl -X POST http://127.0.0.1:5000/order/rox

Observe the robot, Nav2 logs, adapter logs, MQTT state and physical safety system. Release the short-route hold only after arrival is verified:

curl -X POST http://127.0.0.1:5000/release_hold

Acceptance criteria:

exactly one Nav2 goal is issued for the destination;

the robot reaches the correct position and yaw;

the state removes traversed nodes/edges correctly;

the action state changes match the physical event;

cancellation does not cause a delayed goal to start later;

scanner and emergency-stop interventions remain authoritative.

## F6. Generate and test the full ROX-only route

On the Pi:

cd ~/VDA5050-Paper-Dev source .venv/bin/activate python3 scripts/generate_rox_order.py \ --waypoints configs/rox_waypoints.yaml \ --route examples/routes/rox_crane_case_study.yaml \ --output examples/orders/order_rox_diff_v3.json \ --update-fleet-env configs/fleet_control.env ./scripts/run_static_checks.sh

Restart the master and test:

curl -X POST http://127.0.0.1:5000/order/rox

Run the route with the crane powered down or otherwise prevented from moving. Manually release the ROX handover hold after checking alignment.

# Part G — Configure and test the crane adapter

## G1. Install crane Python dependencies on its runtime device

If the crane adapter runs on the Pi:

cd ~/VDA5050-Paper-Dev python3 -m venv .venv-crane source .venv-crane/bin/activate python -m pip install --upgrade pip python -m pip install -r crane_edge/requirements.txt

## G2. Configure credentials without committing them

Preferred environment variables:

export CRANE_OPCUA_URL='<REAL_OPCUA_URL>' export CRANE_ACCESS_CODE='<REAL_NUMERIC_ACCESS_CODE>'

Or create the ignored local file:

cd ~/VDA5050-Paper-Dev/crane_edge cp access.txt.example access.txt chmod 600 access.txt nano access.txt

The file format is:

<OPC_UA_URL> <NUMERIC_ACCESS_CODE>

Never commit or distribute access.txt.

Critical public-repository remediation before continuing: rotate/revoke the previously published crane access code, remove crane_edge/access.txt from Git tracking, add the credential paths to .gitignore, and purge the secret from repository history. Merely deleting the file in a new commit does not remove it from earlier commits. Coordinate any history rewrite with collaborators and make fresh clones afterward.

cd ~/VDA5050-Paper-Dev
git rm --cached --ignore-unmatch crane_edge/access.txt
printf "\n# Local credentials\ncrane_edge/access.txt\naccess.txt\naccesscode_url.txt\n" >> .gitignore
git status --short
# Rotate the credential at the crane/PLC side before using the adapter.
# Use git-filter-repo or an approved equivalent to purge the old secret from history.

Never paste the replacement access code into a command history, document, issue, chat log or committed environment file. Prefer protected environment injection or a mode-600 local file that is confirmed untracked.

## G3. Verify the crane factsheet template

nano ~/VDA5050-Paper-Dev/crane_edge/factsheets/ilmatar_crane_factsheet.template.json

Replace zero/placeholder physical values only with verified values from the actual crane and implementation. Keep the identity aligned with:

manufacturer=konecranes serialNumber=ilmatar_1 version=3.0.0

Validate after editing:

cd ~/VDA5050-Paper-Dev source .venv/bin/activate python3 scripts/validate_messages.py \ --schema-dir schemas/vda5050_v3 \ factsheet crane_edge/factsheets/ilmatar_crane_factsheet.template.json

## G4. Start the crane adapter fail-closed

The crane must be in the approved automatic state and homing/preflight must succeed. Do not set the override for normal operation.

cd ~/VDA5050-Paper-Dev export VDA_MQTT_HOST=192.168.50.115 export ALLOW_UNHOMED_START=false ./scripts/run_crane_adapter.sh

If the adapter exits because automatic mode or homing is not valid, correct the physical/control state. ALLOW_UNHOMED_START=true is for supervised telemetry diagnosis only and must not be used for motion commissioning.

## G5. Test the crane alone

On the Pi master terminal host:

curl -X POST http://127.0.0.1:5000/factsheet/crane curl -X POST http://127.0.0.1:5000/order/crane

Verify exact action IDs from examples/orders/order_ilmatar_v3.json:

python3 - <<'PY' import json p=json.load(open('examples/orders/order_ilmatar_v3.json')) for n in p['nodes']: for a in n.get('actions', []): print(n['nodeId'], a['actionId'], a['actionType']) PY

Acceptance criteria:

wrong participant identity, map or order update is rejected;

a second order is rejected while one is active;

unsupported edge actions are rejected;

node and edge state lists clear as traversal completes;

action states correspond to real PLC/crane events;

factsheet is retained and returned on request;

free-text information[] is display-only and never controls release logic.

# Part H — Coordinated ROX–crane no-load test

Run this only after the full ROX-only route and crane-only order both pass.

## H1. Confirm orchestration IDs

On the Pi:

curl http://127.0.0.1:5000/runtime ¦ python3 -m json.tool

Confirm:

CRANE_AUTO_RELEASE_ACTION_ID = action4 CRANE_MANUAL_RELEASE_ACTION_ID = action6 CRANE_SAFE_LIFT_ACTION_ID = action7 ROX_HOLD_ACTION_ID = rox_hold_at_crane

These values must match the actual crane order and generated ROX route.

## H2. Start all components

1.  Pi Mosquitto.

2.  Pi master control.

3.  Verify the native ROX bringup already started by ROS_AUTOSTART.sh; do not launch a second instance.

4.  ROX Nav2/localization.

5.  ROX VDA adapter in real mode.

6.  Crane VDA adapter in fail-closed mode.

7.  MQTT monitor.

## H3. Send the combined order

On the Pi:

curl -X POST http://127.0.0.1:5000/order

Expected handover logic:

1.  ROX reaches node2 and its exact rox_hold_at_crane action is active.

2.  The master only associates the configured crane action IDs with that rendezvous.

3.  Automatic release uses only action4 when the configured conditions are met.

4.  Manual /release arms only the configured action6 path and expires after the configured TTL.

5.  ROX remains held until the exact crane safe-lift milestone action7 finishes.

6.  A failed action7 does not release the ROX.

7.  Crane hoist telemetry in information[] may be displayed but is not a release condition.

Manual release command, only when the local procedure permits it:

curl -X POST http://127.0.0.1:5000/release

Emergency or abnormal behavior must be handled through the physical safety controls first. Software cancellation is secondary:

curl -X POST http://127.0.0.1:5000/cancel/rox curl -X POST http://127.0.0.1:5000/cancel/crane

Repeat the coordinated test without load until action/state sequencing, alignment and departure are consistently correct. Record logs before considering a supervised loaded test.

# Part I — Optional systemd services after manual tests pass

## I1. Pi master service

sudo cp ~/VDA5050-Paper-Dev/deploy/systemd/vda5050-master.service.example \ /etc/systemd/system/vda5050-master.service sudo nano /etc/systemd/system/vda5050-master.service sudo systemctl daemon-reload sudo systemctl enable --now vda5050-master.service sudo systemctl status vda5050-master.service --no-pager journalctl -u vda5050-master.service -f

Edit the service username and paths to match the real Pi account before enabling it.

## I2. ROX dry-run service first

sudo cp ~/Projects/VDA5050-Paper-Dev/deploy/systemd/rox-vda5050-adapter-dry.service.example \ /etc/systemd/system/rox-vda5050-adapter.service sudo nano /etc/systemd/system/rox-vda5050-adapter.service sudo systemctl daemon-reload sudo systemctl enable --now rox-vda5050-adapter.service sudo systemctl status rox-vda5050-adapter.service --no-pager journalctl -u rox-vda5050-adapter.service -f

Use the real-mode service template only after all manual dry-run and real short-route checks pass.

# Part J — Fast troubleshooting

## Master says schema or order file is missing

cd ~/VDA5050-Paper-Dev ls schemas/vda5050_v3/order.schema ls examples/orders/order_ilmatar_v3.json ls examples/orders/order_rox_diff_v3.json ./scripts/run_static_checks.sh

The ROX order is intentionally generated after waypoint capture.

## DTLabOpen is connected but the Pi or ROX is unreachable

On both devices, verify the DTLabOpen addresses and the route selected for the peer. On the Pi, also confirm that the separate Ilmatar Wi-Fi connection still behaves as intended:

# ROX
ip -br address
ip route
ip route get 192.168.50.115
ping -c 3 192.168.50.115
nc -vz 192.168.50.115 1883

# Pi
ip -br address
ip route
ip route get 192.168.50.50
ping -c 3 192.168.50.50
ss -ltnp | grep 1883

Because both devices are direct DTLabOpen peers, one-way-only reachability is not expected. Check subnet prefixes, duplicate addresses, host firewalls and interface-specific routes on both devices.

## ROX cannot connect to MQTT

ping -c 3 192.168.50.115 nc -vz 192.168.50.115 1883 mosquitto_pub -h 192.168.50.115 \ -t vda5050/v3/commissioning/ping -m test

On Pi:

sudo journalctl -u mosquitto -n 100 --no-pager ss -ltnp ¦ grep 1883

## Adapter cannot find Nav2

ros2 action list -t ¦ grep navigate_to_pose ros2 node list ros2 lifecycle nodes

Start native rox_navigation and confirm it is active before the adapter.

## Order is rejected at the first node

ros2 run tf2_ros tf2_echo map base_link cat ~/VDA5050-Paper-Dev/configs/rox_waypoints.yaml

Physically/localize the robot at the captured first node or regenerate the route with the correct home. Do not increase tolerance merely to hide localization or mapping errors.

## initializePosition does not visibly move the robot

That action initializes localization; it is not a movement command. Verify /initialpose, AMCL/localization and RViz. Then send a normal Nav2 or VDA order.

## Crane adapter exits during startup

Check automatic mode, homing/preflight, OPC UA URL/access code and PLC connectivity. Do not bypass a failed preflight for motion.

## An action is stuck

Inspect exact action IDs and statuses:

curl http://127.0.0.1:5000/runtime ¦ python3 -m json.tool mosquitto_sub -h 127.0.0.1 -t 'vda5050/v3/#' -v

Use cancelOrder, retry or skipRetry only where the adapter reports the corresponding supported state and the physical process permits it.

# Appendix A — Site Configuration Checklist

Complete this file on the real equipment. Do not replace unknown values with guesses.

## Raspberry Pi and network

☐ Pi username: raspberrypi / actual: ____________________

☐ Pi hostname: raspberrypi / actual: ____________________

☐ Pi eth0 / DTLabOpen IPv4: 192.168.50.115 / prefix: ____________________

☐ Pi wlan0 / Ilmatar IPv4: 192.168.0.116 / prefix: ____________________

☐ DTLabOpen gateway: 192.168.1.1

☐ Pi default route uses Ilmatar; DTLabOpen is never-default.

☐ Pi project path: ~/VDA5050-Paper-Dev

☐ MQTT/Flask address: 192.168.50.115:1883 / :5000

☐ ROX DTLabOpen IPv4: 192.168.50.50 / prefix: ____________________

☐ DTLabOpen connection/interface on ROX: ____________________

☐ DTLabOpen gateway observed on ROX: ____________________

☐ DTLabOpen gateway observed on Pi Ethernet: ____________________

☐ Pi route to 192.168.50.50 uses eth0.

☐ ROX route to 192.168.50.115 uses the DTLabOpen-facing interface.

☐ Engineering PC addresses/reservations: ____________________

☐ Pi and ROX can ping each other bidirectionally.

☐ ROX can open TCP 1883 and TCP 5000 on 192.168.50.115.

☐ ROX can open TCP connection to MQTT port 1883.

☐ ROX commissioning MQTT message is observed on the Pi.

☐ Pi-to-ROX management path (if required): port-forward / static route / direct interface / not required.

☐ Anonymous MQTT is blocked on Ilmatar or explicitly approved.

☐ Authentication/TLS plan documented for non-lab use.

## Neobotix delivered software

☒ ROS distribution: jazzy; middleware: rmw_cyclonedds_cpp; ROS_DOMAIN_ID=0

☒ Neobotix workspace: /home/neobotix/ros2_workspace

☒ Native bringup starts automatically at robot boot through ROS_AUTOSTART.sh

☒ ROS_AUTOSTART.sh: source ~/ros2_workspace/install/setup.bash; sleep 2; launch bringup_launch.py with diff, IMU, D435 and I/O board enabled

☒ rox_navigation mapping launch file: mapping.launch.py

☐ Optional remapping command, only if df_map is unsuitable: ros2 launch rox_navigation mapping.launch.py

☒ Navigation launch file: navigation.launch.py; use map:=/home/neobotix/ros2_workspace/install/rox_navigation/share/rox_navigation/maps/df_map.yaml

☒ Navigation robot type must be rox_type:=diff

☒ frame_type is not accepted by the installed launch files; do not pass it

☒ Robot namespace: empty (not explicitly passed)

☒ Lidar: /scan — sensor_msgs/msg/LaserScan

☒ Odometry: /odom — nav_msgs/msg/Odometry

☒ Battery: /battery_state — sensor_msgs/msg/BatteryState

☒ Emergency stop: /emergency_stop_state — neo_msgs2/msg/EmergencyStopState

☒ Safety state: /safety_state — neo_msgs2/msg/SafetyState

☐ Nav2 action absent during native bringup; record after navigation starts: ____________________

☐ Base frame: base_link expected; verify from active TF tree: ____________________

☐ Map frame: map expected; verify after localization/navigation: ____________________

## Map and navigation

☒  Map ID: df_map

☒  Map YAML absolute path: /home/neobotix/ros2_workspace/install/rox_navigation/share/rox_navigation/maps/df_map.yaml

☒  Map image absolute path: /home/neobotix/ros2_workspace/install/rox_navigation/share/rox_navigation/maps/df_map.pgm

☐  df_map.yaml and df_map.pgm copied together outside the workspace install tree; backup path: ____________________

☐  Lidar alignment verified in RViz.

☐  Robot footprint and payload footprint verified.

☐ Global/local costmaps verified on df_map; NavigateToPose action is listed after navigation startup.

☐ Controller/planner verified; map -> base_link TF is stable after localization on df_map.

☐  Commissioning maximum linear speed: ____________________

☐  Commissioning maximum angular speed: ____________________

☐  Emergency stops accessible throughout test route.

## Captured waypoints

| Waypoint | x (m) | y (m) | theta (rad) | XY tolerance (m) | theta tolerance (rad) | Repeated Nav2 test passed |
| --- | --- | --- | --- | --- | --- | --- |
| home |  |  |  |  |  | [ ] |
| short_test |  |  |  |  |  | [ ] |
| crane_handover |  |  |  |  |  | [ ] |
| warehouse_dropoff |  |  |  |  |  | [ ] |

☐  All waypoints belong to the same saved map.

☐  configs/rox_waypoints.yaml changed to configured: true only after verification.

☐  Short order generated and validated.

☐  Full route generated and validated.

## VDA identities and topics

☐  Version: 3.0.0

☐  ROX manufacturer: neobotix

☐  ROX serial number: rox_diff_1 / actual configured: ____________________

☐  ROX topic root: ____________________

☐  Crane manufacturer: konecranes

☐  Crane serial number: ilmatar_1 / actual configured: ____________________

☐  Crane topic root: ____________________

☐  Factsheets contain verified physical/capability values.

☐  Factsheet identities match adapter/master identities exactly.

## Crane and handover

☐  OPC UA URL configured outside Git.

☐  Access code configured outside Git.

☐  Crane automatic mode requirement verified.

☐  Homing/preflight requirement verified.

☐  ALLOW_UNHOMED_START=false for motion tests.

☐  Crane handover node ID: ____________________

☐  ROX handover node ID: ____________________

☐  Automatic release action ID: ____________________

☐  Manual release action ID: ____________________

☐  Safe-lift completion action ID: ____________________

☐  ROX hold action ID: ____________________

☐  Manual release time-to-live: ____________________

☐  Exact action IDs checked against both order files.

☐  Free-text information[] excluded from safety/control decisions.

☐  Crane-only test passed without load.

☐  ROX-only full route passed without crane motion.

☐  Coordinated no-load test passed repeatedly.

☐  Failure/recovery procedure documented for broker loss, adapter loss, localization loss, Nav2 failure and scanner intervention.

## Repository and credential hygiene

☐ Exact Git commit recorded for the Pi and ROX checkouts.

☐ crane_edge/access.txt is not returned by git ls-files.

☐ Credential ignore rules are active in .gitignore.

☐ Previously published crane credential has been rotated/revoked.

☐ Repository history has been purged or access risk has been formally accepted and documented.

☐ No secrets are present in git status, git diff, documentation or commissioning logs.

☐ Static checks pass after repository changes.

## Sign-off

Test date: ____________________

Software commit/package: ____________________

Operator: ____________________

Observer/safety responsible person: ____________________

Result and open issues: ____________________________________________________________